Legend Hobby Shock Cub Navigation Lights

The Legend Hobby Shock Cub Navigation Light Set is designed to plug into any spare channel on your receiver and provides two 8mm Wingtip Navigation Lights (1 x red/1 x green), two Asynchronous white strobes, and two Landing Lights.  All the lights can be switched on/off from the transmitter.

In order to allow the various outputs to be switched from the transmitter, the circuit should be plugged into a proportional channel, typically channel 6 or higher.  Avoid using the gear channel unless you are certain that your radio allows proportional control of the gear channel.  Adjust the ATV or End Point of the channel you choose to 90-95% in each direction, but no more than 100%.  The channel you select should be one that can be controlled with a knob or slider, and/or a 2 or 3-position switch.  This Shock Cub Navigation Light Set has 3 modes, which respond to approximately the programmable channel mixes shown below:

Mode #1:  All Lights Off------------------------------------0% or -100%)

Mode #2:  Navigation and Strobes On---------------- greater than 25% (-50%)

Mode #3:  All Lights, including Landing lights, On-- greater than 50% (0% or channel center)

To use all three Modes, assign a 3-position switch (or use a knob or slider) to move the channel you use for your lights to the approximate channel settings shown in the chart above.  If you prefer to have only two Modes, (Mode #2 and Mode #3), you can raise the lower end point (ATV) of the channel you are using to approximately -50%, or until the Simulated Rotating beacons come on along with the wingtip lights.  Assign a switch to the channel you’re using, and set it to raise the channel’s upper end point to above the channel’s center.  This will give you control of the Landing Lights only, but will allow the use of a 2-position switch instead of a 3-position switch.  To make setup easier, changes to the channel’s end points can be made on the fly.

If you have any questions or problems, don’t hesitate to contact me.  ENJOY!

www.davesrce.com

sales@davesrce.com

SCAN HERE

(423) 544-1657